# Tamil Nadu Hydropower
PyPSA Implementation Note

Reservoir representation, data mapping, and implementation guidance for Codex

Purpose. This note translates the available Tamil Nadu hydropower and reservoir data into a practical PyPSA modelling approach. It is written as an implementation brief: Codex should use the recommendations below when adding conventional reservoir hydro, cascades, run-of-river plants, and pumped storage to the Tamil Nadu model.

# 1. PyPSA modelling recommendation

PyPSA provides two relevant abstractions. A StorageUnit combines turbine power and energy storage in one component through max_hours = energy_capacity / power_capacity. It directly supports exogenous hydro inflow and endogenous spillage. A Store + Link formulation separates reservoir energy capacity from turbine/pump power and is therefore better when reservoirs and powerhouses form cascades or when power and storage capacities are not intrinsically coupled.

| Hydro type | Recommended PyPSA representation | Why | Tamil Nadu examples |
| --- | --- | --- | --- |
| Single reservoir, one main powerhouse | StorageUnit | Simple; native inflow, spill and state_of_charge | Mettur, Bhavanisagar, Sholayar, Amaravathi, Periyar, Vaigai, Papanasam |
| Cascade / shared reservoirs | Store + water Bus + Link(s) | Separates reservoir MWh from turbine MW and allows water to pass downstream | Kundah cascade, Parambikulam-Aliyar system, Kodayar cascade |
| Run-of-river / barrage | Generator with time-varying p_max_pu, or Link from upstream water bus | No meaningful seasonal storage | Lower Mettur barrages, Bhavani Kattalai, Bhavani barrages |
| Pumped storage | Store + two Links (turbine and pump) | Independent pump/turbine efficiencies and reservoir capacity; explicit charging | Kadamparai PSP |

# 2. StorageUnit formulation for simple reservoirs

Use StorageUnit when a plant can be reasonably represented as one reservoir connected to one electrical bus and the reservoir energy capacity can be expressed as equivalent full-load hours. PyPSA defines e_nom = p_nom × max_hours. The native energy balance is conceptually:

soc[t] = standing_eff × soc[t-1]
       + efficiency_store × p_store[t]
       - p_dispatch[t] / efficiency_dispatch
       - spill[t] + inflow[t]

For conventional reservoir hydro, pumping should not be available. The exogenous inflow time series adds energy to the reservoir; dispatch removes stored energy; and spill is available when inflow exceeds usable storage.

n.add("StorageUnit",
      "Mettur_hydro",
      bus="Tamil_Nadu",
      carrier="reservoir_hydro",
      p_nom=250,
      max_hours=E_reservoir_MWh / 250,
      efficiency_dispatch=eta_turbine,
      efficiency_store=1.0,
      standing_loss=0.0,
      cyclic_state_of_charge=False)

n.storage_units_t.inflow["Mettur_hydro"] = inflow_MW_series

Do not automatically set cyclic_state_of_charge=True for seasonal hydro. A cyclic annual constraint can artificially force the model to end the year at the starting reservoir level. Prefer an observed initial state_of_charge and, where possible, an observed end-of-horizon constraint or a multi-year/rolling-horizon method. PyPSA also provides a water-value example for long-duration storage when limited foresight is used.

# 3. Store + Link formulation for cascades

For the Kundah and Parambikulam-Aliyar systems, use a hydraulic-energy bus for each reservoir, a Store for stored water expressed in MWh-equivalent, and Links for each turbine or transfer. This follows the more fundamental PyPSA formulation and allows the upstream turbine discharge to become downstream inflow.

# Reservoir energy carrier
n.add("Bus", "Upper_Bhavani_water", carrier="water_energy")
n.add("Store", "Upper_Bhavani_reservoir",
      bus="Upper_Bhavani_water",
      e_nom=E_upper_bhavani_MWh,
      e_initial=E_initial_MWh,
      e_cyclic=False)

# Natural inflow represented as an exogenous source to the water bus
n.add("Generator", "Upper_Bhavani_inflow",
      bus="Upper_Bhavani_water", carrier="rain",
      p_nom=inflow_MW.max(),
      p_max_pu=inflow_MW / inflow_MW.max(),
      marginal_cost=0)

# Turbine converts stored hydraulic energy to electricity
n.add("Link", "Kundah_V_turbine",
      bus0="Upper_Bhavani_water", bus1="Tamil_Nadu",
      p_nom=P_input_equiv_MW,
      efficiency=eta_turbine)

For a true cascade, connect the turbine discharge to the next reservoir rather than losing it. If the hydraulic energy basis changes between reservoirs because the usable head changes, use a multi-port Link or an explicit conversion factor so that the downstream water-energy quantity is consistent. If travel time matters, PyPSA Link transport-delay functionality can represent delayed arrival; otherwise use zero delay.

# 4. Converting reservoir data to PyPSA energy

Preferred approach: use measured stored-energy data directly when available. SRPC reports several Nilgiris reservoirs in MU. Since 1 MU = 1,000 MWh = 1 GWh, these series can directly define reservoir energy state after consistent unit conversion.

When only live storage volume V is available, convert usable water to electrical energy with:

E [MWh] = ρ × g × V [m³] × H [m] × η / 3.6×10⁹

Use live/active storage rather than gross storage whenever possible. H should be effective hydraulic head. If head varies strongly with reservoir level, a constant-head conversion is an approximation and should be documented. Where SRPC already supplies stored energy, do not recompute it from volume.

# 5. Inflow time series

PyPSA expects StorageUnit.inflow in MW-equivalent energy per snapshot. For hourly snapshots, an inflow of 100 MW contributes 100 MWh during one hour. Convert observed volumetric inflow Q to electrical-equivalent inflow using the same head and conversion efficiency used for the storage representation. For daily data, preserve daily energy before disaggregating to hourly values.

If daily inflow is reported as volume, convert daily volume to MWh/day and divide by 24 for a flat hourly profile unless a higher-resolution inflow profile exists. Do not interpolate reservoir state itself and then derive inflow without accounting for releases, generation, evaporation and spill.

# 6. Initial and boundary conditions

Set the initial reservoir state from observed storage on the first model day whenever available.

Avoid assuming reservoirs start full.

Avoid annual cyclicity for seasonal reservoirs unless the study explicitly requires a representative-year equilibrium.

For rolling-horizon operation, consider a terminal water value or terminal storage target to prevent artificial depletion near the horizon boundary.

Validate simulated state_of_charge against observed reservoir storage/energy for historical years.

# 7. Tamil Nadu plant-to-data mapping

| Plant/system | MW | Reservoir/source | Best data | PyPSA object | Status |
| --- | --- | --- | --- | --- | --- |
| Mettur Dam + Tunnel | 250 | Mettur / Stanley | TN Agriculture daily: level, storage, inflow, outflow | StorageUnit | Ready |
| Lower Mettur Barrage I-IV | 120 | Cauvery below Mettur | Mettur outflow as hydro driver | Generator / cascade Link | Proxy |
| Bhavanisagar / Lower Bhavani | 16 | Bhavanisagar | TN Agriculture daily + CWC | StorageUnit | Ready |
| Sholayar I-II | 109 | Sholayar | TN Agriculture daily + CWC | StorageUnit or PAP Store | Ready |
| Sarkarpathy | 30 | Thunacadavu fed from Parambikulam | Parambikulam daily; Thunacadavu gap | Store + Link | Partial |
| Aliyar PH | 60 | Upper Aliyar → Aliyar | Aliyar daily; Upper Aliyar gap | Store + Link | Partial |
| Kadamparai PSP | 400 | Kadamparai ↔ Upper Aliyar | CEA static reservoir parameters; no continuous public series found | Store + pump Link + turbine Link | Data gap |
| Amaravathi | 4 | Amaravathi | TN Agriculture daily | StorageUnit | Ready |
| Periyar | 168 | Periyar | TN Agriculture daily | StorageUnit | Ready |
| Vaigai | 6 | Vaigai | TN Agriculture daily + CWC | StorageUnit | Ready |
| Papanasam | 32 | Papanasam | TN Agriculture daily | StorageUnit | Ready |
| Kodayar I-II | 100 nominal | Upper/Lower Kodayar → Pechiparai | Static reservoir parameters; Pechiparai downstream data | Store + Link | Partial |
| Kundah V | 40 | Upper Bhavani | SRPC monthly level, inflow, stored energy | Store + Link | Good |
| Kundah I | 60 | Avalanche + Emerald | SRPC monthly stored energy/inflow | Store + Link | Good |
| Kundah II-IV/VI | 485 | Kundah/Pegumbahalla/Pillur/Parsons Valley | Static storage; incomplete operational time series | Cascade Store + Link | Partial |
| Pykara + Micro | 61.2 | Pykara | SRPC monthly stored energy/inflow | Store + Link | Good |
| PUSHEP | 150 | Pykara New Forebay / Glenmorgan | Static data; operating series incomplete | Cascade Store + Link | Partial |

# 8. Data priority for implementation

1. Build daily historical series from the Tamil Nadu Agriculture reservoir database for all covered reservoirs.

2. Extract SRPC monthly inflow and stored-energy series for Pykara, Upper Bhavani, Avalanche/Emerald and Mukurthy.

3. Convert each storage series to MWh-equivalent and calculate max_hours only for plants modelled as StorageUnit.

4. Create explicit reservoir Stores and turbine Links for the Kundah, PAP and Kodayar cascades.

5. Keep Kadamparai separate as pumped storage; do not merge it with conventional hydro inflows.

6. Represent barrage/run-of-river plants with time-varying generation availability derived from upstream releases or inflows.

7. Leave missing operational reservoir time series blank or flagged; do not fabricate them.

# 9. Validation checks Codex should implement

Installed turbine MW in PyPSA equals the selected source capacity.

Reservoir e_nom or max_hours matches the chosen live-storage energy capacity.

Annual sum of inflow MWh is physically plausible relative to observed annual hydro generation.

state_of_charge never exceeds e_nom or becomes negative.

Historical simulated reservoir trajectory is compared with observed storage for at least one calibration year.

Spillage is inspected; persistent large spill may indicate an incorrect inflow conversion or too-small energy capacity.

Cascade energy is conserved through successive reservoirs and turbine Links.

Pumped-storage charging and generation are kept distinct from natural hydro inflow.

# 10. Suggested data schema

| File | Key fields | Purpose |
| --- | --- | --- |
| hydro_plants.csv | name, bus, p_nom_mw, hydro_type, reservoir_id, downstream_reservoir_id, efficiency | Static plant mapping |
| hydro_reservoirs.csv | reservoir_id, e_nom_mwh, source_volume_mcm, head_m, initial_soc_mwh, source | Reservoir parameters |
| hydro_inflow.csv | snapshot + one column per reservoir/StorageUnit | Time-varying natural inflow in MW |
| hydro_observed_soc.csv | snapshot + observed reservoir storage/energy | Calibration and validation |
| hydro_links.csv | name, reservoir_from, reservoir_to/electric_bus, p_nom_mw, efficiency, delay_h | Cascade and pumped-storage Links |

# 11. Sources

PyPSA documentation — StorageUnit: https://docs.pypsa.org/latest/user-guide/components/storage-units/

PyPSA documentation — Store: https://docs.pypsa.org/latest/user-guide/components/stores/

PyPSA example — StorageUnit as Link and Store components: https://docs.pypsa.org/latest/examples/replace-generator-storage-units-with-store/

PyPSA examples overview — Chained Hydro Reservoirs: https://docs.pypsa.org/latest/examples/examples/

PyPSA example — Water Values for Long-Duration Storage Operation: https://docs.pypsa.org/latest/examples/water-value/

Tamil Nadu Agriculture reservoir database: https://tnagriculture.in/ARS/home/reservoir/

Central Water Commission Reservoir Storage Monitoring System: https://rsms.cwc.gov.in/

Southern Region Power Committee annual reports: https://srpc.kar.nic.in/

Central Electricity Authority pumped-storage documentation: https://cea.nic.in/

# 12. Implementation decision summary

Recommended default: use StorageUnit for isolated/simple conventional reservoirs, but use Store + Link for Tamil Nadu's cascaded systems and for Kadamparai pumped storage. This preserves the physical distinction between reservoir energy capacity, turbine capacity, pumping capacity and downstream water routing.